GUIDANCE NOTE:		Inadvertent Exposure to Asbestos Containing Materials

___________________________________________________________________________________

If you inadvertently disturb a material which you suspect may contain asbestos, or if you consider you may have been exposed to such a material as a result of work carried out by others (maintenance personnel/contractors) in your area, then:

- stop that work

- isolate, lock off the area of potential exposure, so far as is reasonably practicable, and place appropriate warning signs

- inform your Building Liaison Officer (BLO), Trade Union Safety Representative and your immediate Supervisor.

The BLO, Safety Representative and/or the Supervisor will consult the asbestos register and if there is any doubt about the nature of the material disturbed, they will inform the Environmental Manager (by phone, fax or via the Help Desk 5152).

The Environmental Manager will also consult the asbestos register and visit the scene of the incident.  If there remains any doubt about the material and whether it contains asbestos or not, the Environmental Manager will appoint a UKAS accredited Analyst to examine the material disturbed.

If the material is found not to contain asbestos:

The Environmental Manager will inform those parties above to that effect and issue them with a copy of the Analyst’s report and declare that area is safe for re-occupation.

If the material is found to contain asbestos:

The Environmental Manager will:

- inform the parties that they may have been exposed to asbestos.  Their line management should also be informed,

- instruct the Analyst to carry out air monitoring in the contaminated area prior to remedial action being taken to assess potential exposures,

- inform the University Safety Service (USS) about the incident,

- appoint a licensed Asbestos Contractor to repair or remove the asbestos disturbed and to decontaminate the area,

- ensure appropriate air monitoring is conducted by the Analyst during the remedial work,

- on completion of the work obtain a clearance certificate for the area from the Analyst stating that it is safe to re-occupy,

- inform the relevant parties when the area has been declared safe and supply them with a copy of the Analyst’s report,

- introduce any measures recommended by the USS to prevent a recurrence

- review the University’s procedures on work with asbestos containing materials as appropriate,

- amend the asbestos register as appropriate,

- 2 -

- inform the Estates Services Managers and Estates Managers of the relevant changes to procedures and the register.

The University Safety Service will investigate the incident:

- to determine those potentially exposed,

- to determine the nature of the asbestos disturbed,

- to estimate the possible magnitude and time of exposure.

The University Safety Service will:

- make recommendations to the Environmental Manager and Area Supervisor as appropriate to prevent a recurrence,

- report the incident to the Occupational Health Service.

The Occupational Health Service will:

- arrange appointments for those potentially exposed to attend the Occupational Health Physician,

- provide counselling,

- maintain health records on those potentially exposed for 40 years.

The line management will:

- ensure all staff are warned to stay out of the contaminated area,

- inform staff when it is safe to re-enter the area

- introduce any measures recommended by the USS to prevent a recurrence.

6 February 2004